DROWSINESS DETECTOR

Master's Project Documentation

Day 3 — EAR Formula, Threshold Logic & Drowsy Alert

| Day | 3 of 7–14 |
| --- | --- |
| Topic | Eye Aspect Ratio — Formula, Threshold, Frame Counter, Alert |
| New file | src/ear_detector.py |
| Goal | Close eyes 2 sec — red DROWSY alert fires on screen and in terminal |
| Builds on | Day 2 — face_landmarks.py must show 468 landmarks before starting Day 3 |

Software Engineering | Machine Learning | Computer Vision

# 1. Academic Context — Where EAR Comes From

EAR was not invented for this project — it comes from peer-reviewed academic research. This section gives you the literature background to cite in your presentation. One sentence from this section, delivered confidently to your professors, immediately elevates your work.

## 1.1 The Original Paper

Citation: Soukupova, T. and Cech, J. (2016). Real-Time Eye Blink Detection using Facial Landmarks. Czech Technical University in Prague. Centre for Machine Perception.

URL: https://vision.fe.uni-lj.si/cvww2016/proceedings/papers/05.pdf

What the paper introduced:

- The EAR formula as a single scalar measure of eye openness

- Validation that EAR drops sharply and consistently during eye closure

- The concept of a fixed threshold combined with a temporal frame counter

- An SVM classifier trained on EAR sequences for blink detection

| Present it as | "The Eye Aspect Ratio was introduced by Soukupova and Cech at Czech Technical University in 2016 as a lightweight geometric feature for real-time eye blink detection. I have adapted their formula for drowsiness detection using MediaPipe's 468-point face mesh rather than the dlib 68-point model used in the original paper." |
| --- | --- |

Why your implementation is different from the original paper:

| Aspect | Original paper (2016) | Your implementation (2026) |
| --- | --- | --- |
| Face model | dlib 68-point shape predictor | MediaPipe FaceMesh 468-point |
| Eye landmarks | 6 specific dlib indices (36–41) | 6 MediaPipe indices per eye |
| Performance | Slower — not mobile-optimised | 25+ fps on CPU — mobile-first |
| Classification | SVM on EAR sequences | CNN + geometric threshold |
| Target | Blink detection | Drowsiness detection (longer closure) |

## 1.2 Supporting 2024 Paper

Citation: "Real-Time Drowsiness Detection Using Eye Aspect Ratio and Facial Landmark Detection" (2024). arxiv.org/abs/2408.05836

URL: https://arxiv.org/abs/2408.05836

This more recent paper uses the same approach as yours: EAR + MediaPipe + threshold + frame counter. Key findings you can cite:

- EAR threshold of 0.25 is validated as effective across diverse subject demographics

- 20-frame (0.66 second) window is sufficient to distinguish drowsiness from normal blinking

- The system achieves reliable detection with low computational demands on CPU-only hardware

| Academic strength | Having two citations — a 2016 foundational paper and a 2024 validation paper — shows literature awareness. Most student projects cite zero papers. Yours cites two. This is a meaningful differentiator in a master's presentation. |
| --- | --- |

# 2. Learning Sources — Study Before You Code

Read these in order before opening VS Code. Targeted reading only — the specific sections listed, not full courses.

## 2.1 The EAR Formula — Original Paper Section 2

What to read: Section 2 only — "Eye Aspect Ratio". Read it twice. Look at Figure 1.

URL: https://vision.fe.uni-lj.si/cvww2016/proceedings/papers/05.pdf

Time: 20 minutes

- Understand the 6-point eye model: P1 (left corner), P4 (right corner), P2/P3 (upper lid), P5/P6 (lower lid)

- Understand why the formula divides by 2 x horizontal distance — normalisation for face size

- Understand why EAR is constant when eyes are open and drops sharply when closed

## 2.2 Euclidean Distance with NumPy

What to read: The np.linalg.norm() section only. Stop after the first working example.

URL: https://www.geeksforgeeks.org/python/calculate-the-euclidean-distance-using-numpy/

Time: 15 minutes

The one thing to understand: np.linalg.norm(point_a - point_b) computes the straight-line distance between two NumPy arrays. Internally it does sqrt(sum(squared_differences)). For 2D points it is identical to Pythagoras' theorem.

import numpy as np # Pythagoras manually: # distance = sqrt( (x2-x1)^2 + (y2-y1)^2 ) # = sqrt( (3-0)^2 + (4-0)^2 ) # = sqrt( 9 + 16 ) = sqrt(25) = 5.0 p1 = np.array([0, 0]) # origin p2 = np.array([3, 4]) # 3-4-5 right triangle dist = np.linalg.norm(p1 - p2) print(dist) # → 5.0 (verify this is correct!) # This is EXACTLY what your euclidean_distance() function does # with eye landmark pixel coordinates instead of (0,0) and (3,4)

## 2.3 LearnOpenCV — Drowsiness Detection with MediaPipe

What to read: Full article. This is the most directly relevant tutorial online.

URL: https://learnopencv.com/drowsiness-detection-using-mediapipe/

Time: 30 minutes

Read before coding — not after. Their code is slightly different from yours (they use scipy.spatial.distance.euclidean instead of np.linalg.norm) but the landmark indices and formula are identical. Understanding the differences will help you explain your own implementation choices.

## 2.4 cv2.putText() Reference

What to read: Just the Python Parameters section — 10 minutes.

URL: https://docs.opencv.org/4.x/d6/d6e/group__imgproc__draw.html#ga0f9314ea6e35f99bb23f29567fc16e11

Parameter reference for cv2.putText() — you will call this 5+ times in today's code:

| Parameter | Typical value | What it controls |
| --- | --- | --- |
| img | frame | The image to draw on |
| text | "EAR: 0.31" | The string to display |
| org | (10, 30) | Bottom-left corner of text in pixels |
| fontFace | cv2.FONT_HERSHEY_SIMPLEX | Clean sans-serif, readable at all sizes |
| fontScale | 0.7 | 1.0 = ~30px height. 0.7 is readable without dominating frame |
| color | (0, 255, 0) | BGR tuple. Green=(0,255,0), Red=(0,0,255), White=(255,255,255) |
| thickness | 2 | Line thickness in pixels. 2 is standard for video overlays |

# 3. Core Concepts — Day 3

## 3.1 The EAR Formula — Full Mathematical Breakdown

### The 6 landmark points

For each eye, MediaPipe provides 6 specific points. Their arrangement is consistent across all human faces:

| Point | MediaPipe index (left) | Position on eye | Role in EAR |
| --- | --- | --- | --- |
| P1 | 33 | Far left corner | Horizontal — eye width start |
| P2 | 160 | Upper lid left | Vertical — upper measurement |
| P3 | 158 | Upper lid right | Vertical — upper measurement |
| P4 | 133 | Far right corner | Horizontal — eye width end |
| P5 | 153 | Lower lid right | Vertical — lower measurement |
| P6 | 144 | Lower lid left | Vertical — lower measurement |

### The formula step by step

# ¦P2 - P6¦ + ¦P3 - P5¦ # EAR = ───────────────────────── # 2 x ¦P1 - P4¦ # Step 1: Measure vertical height at two points along the eye vertical_1 = euclidean_distance(P2, P6) # upper-lower at left third vertical_2 = euclidean_distance(P3, P5) # upper-lower at right third # Step 2: Measure horizontal width of the eye horizontal = euclidean_distance(P1, P4) # Step 3: Compute ratio ear = (vertical_1 + vertical_2) / (2.0 * horizontal) # Why two vertical measurements? # The eyelid is not a flat line — it curves. # Measuring at two points and averaging captures the curve shape better # than a single central measurement. # Why divide by 2 x horizontal? # Normalisation — makes EAR independent of face size and distance. # A face 30cm from camera and 100cm from camera get the same EAR # for the same eye openness.

### EAR values in practice

| State | Typical EAR range | Meaning |
| --- | --- | --- |
| Eyes fully open | 0.28 – 0.35 | Wide awake, alert |
| Eyes relaxed/tired | 0.22 – 0.28 | Fatigued but conscious |
| Normal blink | 0.05 – 0.20 (brief, <12 frames) | Fast involuntary blink — not drowsy |
| Eyes closing (drowsy) | 0.10 – 0.22 sustained | Slow controlled closure — danger |
| Eyes fully closed | 0.00 – 0.08 | Asleep or blinking |
| THRESHOLD | < 0.25 for 20+ frames | DROWSY — alert fires |

## 3.2 Why Average Left and Right EAR?

The system calculates EAR for each eye independently, then averages the two values:

left_ear = calculate_EAR(left_eye_coords) right_ear = calculate_EAR(right_eye_coords) avg_ear = (left_ear + right_ear) / 2.0 # ← always use the average

Reasons for averaging rather than using one eye:

- Natural facial asymmetry — most people have one eye that naturally opens slightly less than the other. A fixed per-eye threshold would cause false alarms on the naturally smaller eye.

- Medical conditions — mild ptosis (drooping eyelid) is common and would permanently depress one eye's EAR below threshold even when fully awake.

- Glasses — thick frames can partially occlude one eye's landmarks, causing anomalous readings. Averaging with the clear eye corrects this.

- Robustness — requires both eyes to be simultaneously low, not just one, which is more consistent with genuine drowsiness.

## 3.3 The Frame Counter — Why 20 and Not Another Number

The frame counter is what separates a drowsiness detector from a blink detector. A blink lasts 0.3–0.4 seconds (9–12 frames at 30fps). Drowsiness causes sustained closure — 0.6 seconds or longer.

| Threshold | Duration at 30fps | Consequence |
| --- | --- | --- |
| 10 frames | 0.33 seconds | Triggers on normal fast blinks — too many false alarms |
| 15 frames | 0.50 seconds | Borderline — catches some blinks, may miss early drowsiness |
| 20 frames (CHOSEN) | 0.66 seconds | 1.65x longer than max blink — confident drowsiness signal |
| 30 frames | 1.00 second | Misses early drowsiness — too permissive |
| 50 frames | 1.67 seconds | Far too slow — dangerous at highway speeds |

| Defense Q | How did you choose the 20-frame threshold? — A normal blink lasts 0.3–0.4 seconds (Stern et al., 1994), equivalent to 9–12 frames at 30fps. The 20-frame threshold of 0.66 seconds is 1.65 times the maximum blink duration, providing a safety margin that eliminates false alarms from fast blinks while remaining responsive to genuine drowsiness onset. |
| --- | --- |

## 3.4 The Three Visual States — Green / Yellow / Red

Your system should have three distinct visual states that the audience can understand instantly without reading any text:

| State | Condition | Visual | What shows on screen |
| --- | --- | --- | --- |
| Safe | EAR >= 0.25 | Green frame border | EAR value, "SAFE" text, counter = 0 |
| Warning | EAR < 0.25, frames 1–19 | Yellow/amber frame border | EAR value, counter climbing, "CAUTION" |
| Drowsy | EAR < 0.25, frames >= 20 | Red banner across top of frame | "DROWSY DETECTED" in large white text |

| Presentation tip | During your demo, talk the audience through the state transitions: 'Right now you can see the green border — EAR is 0.31, driver is alert. I will now close my eyes... watch the counter climbing... and now the red alert fires.' Narrating what they are seeing makes the system logic instantly understandable. |
| --- | --- |

# 4. Day 3 Code — Complete & Annotated

Build this file in steps. Write Step 1, run it, verify it works. Then add Step 2. Never write more than one step without testing. This approach means bugs are always isolated to the last thing you wrote.

## 4.1 File: src/ear_detector.py

### Part A — Imports, constants, and helper functions

# ────────────────────────────────────────────────────────────── # FILE: src/ear_detector.py # PURPOSE: Add EAR formula to live landmark feed. # Display EAR value live. Alert when drowsy. # RUN: python src/ear_detector.py # BUILT ON: face_landmarks.py (Day 2) # ────────────────────────────────────────────────────────────── import cv2 import mediapipe as mp import numpy as np import time # ── MediaPipe aliases ──────────────────────────────────────── mp_face_mesh = mp.solutions.face_mesh mp_drawing = mp.solutions.drawing_utils # ── Eye landmark indices (MediaPipe FaceMesh) ──────────────── # 6 points per eye in the order: P1, P2, P3, P4, P5, P6 # P1=left corner, P4=right corner, P2/P3=upper lid, P5/P6=lower lid # Source: Soukupova & Cech (2016) adapted for MediaPipe indices LEFT_EYE = [33, 160, 158, 133, 153, 144] RIGHT_EYE = [362, 385, 387, 263, 373, 380] MOUTH_IDX = [61, 291, 0, 17, 78, 308] # prepared for Day 4 # ── Detection parameters ───────────────────────────────────── EAR_THRESHOLD = 0.25 # Below this = eyes closing # Chosen because: normal blink = 0.3-0.4s (9-12 frames) # 20 frames = 0.66s = 1.65x max blink duration DROWSY_FRAMES = 20 # Consecutive frames below threshold to trigger alert # ── Colours (BGR format) ───────────────────────────────────── GREEN = (0, 255, 0) YELLOW = (0, 215, 255) RED = (0, 0, 255) WHITE = (255, 255, 255) BLACK = (0, 0, 0)

### Part B — Helper functions (write and unit-test each before moving on)

# ── FUNCTION 1: Euclidean distance ─────────────────────────── def euclidean_distance(point_a, point_b): """ Calculate straight-line distance between two 2D points. Args: point_a: tuple or array (x, y) point_b: tuple or array (x, y) Returns: float: Euclidean distance in the same units as input Test: euclidean_distance((0,0), (3,4)) should return 5.0 (3-4-5 right triangle — verifiable by Pythagoras' theorem) """ return np.linalg.norm(np.array(point_a) - np.array(point_b)) # ── FUNCTION 2: Eye Aspect Ratio ───────────────────────────── def calculate_EAR(eye_coords): """ Calculate Eye Aspect Ratio from 6 landmark coordinates. Formula from: Soukupova & Cech (2016), Czech Technical University. Args: eye_coords: list of 6 (x, y) tuples in order [P1, P2, P3, P4, P5, P6] P1=left corner, P4=right corner P2/P3=upper lid, P5/P6=lower lid Returns: float: EAR value — ~0.30 open, ~0.20 tired, ~0.04 closed """ P1, P2, P3, P4, P5, P6 = eye_coords # Two vertical measurements (height of eye at two x-positions) # These capture the curvature of the eyelid across its span vertical_1 = euclidean_distance(P2, P6) vertical_2 = euclidean_distance(P3, P5) # One horizontal measurement (width of eye, corner to corner) horizontal = euclidean_distance(P1, P4) # Ratio: vertical / horizontal, normalised for face size # Division by 2*horizontal keeps EAR consistent regardless # of how close the subject is to the camera ear = (vertical_1 + vertical_2) / (2.0 * horizontal) return ear # ── FUNCTION 3: Extract eye pixel coordinates ───────────────── def get_eye_coords(face_landmarks, eye_indices, frame_w, frame_h): """ Extract pixel coordinates for a set of landmark indices. Args: face_landmarks: MediaPipe face landmark object eye_indices: list of 6 MediaPipe landmark indices frame_w: webcam frame width in pixels frame_h: webcam frame height in pixels Returns: list of 6 (x_pixel, y_pixel) tuples """ coords = [] for idx in eye_indices: lm = face_landmarks.landmark[idx] # Convert normalised (0.0-1.0) to pixel coordinates # landmark.x is 0.0=left edge, 1.0=right edge of frame x_px = int(lm.x * frame_w) y_px = int(lm.y * frame_h) coords.append((x_px, y_px)) return coords # ── FUNCTION 4: Draw eye contour ────────────────────────────── def draw_eye_contour(frame, eye_coords, colour): """ Draw a polygon connecting the 6 eye landmark points. Colour changes with eye state: green=open, red=closing. """ pts = np.array(eye_coords, dtype=np.int32) # isClosed=True connects the last point back to the first cv2.polylines(frame, [pts], isClosed=True, color=colour, thickness=1) # ── UNIT TEST — run this before the main loop ───────────────── # Verifies the math is correct using a known answer (3-4-5 triangle) assert abs(euclidean_distance((0,0), (3,4)) - 5.0) < 0.001, "Distance formula incorrect" print("euclidean_distance(): PASS (3-4-5 triangle = 5.0)") # Test EAR with a perfectly circular eye (all ratios equal) # A circle with these 6 points should give EAR ≈ 0.30 test_eye = [(180,90),(260,55),(320,65),(340,90),(320,115),(260,125)] test_ear = calculate_EAR(test_eye) print(f"calculate_EAR() test: {test_ear:.3f} (expected ~0.28-0.32)")

### Part C — Initialisation (outside the loop)

# ── Initialise MediaPipe FaceMesh ──────────────────────────── # OUTSIDE the loop — initialising inside would reload the neural # network 30 times per second, dropping fps from 25 to 1-2. face_mesh = mp_face_mesh.FaceMesh( static_image_mode = False, # Video mode — temporal reuse max_num_faces = 1, # One driver refine_landmarks = True, # Include iris points for Day 8 min_detection_confidence = 0.5, min_tracking_confidence = 0.5 ) # ── Open webcam ────────────────────────────────────────────── # Change to VideoCapture(1) for DroidCam cap = cv2.VideoCapture(0) if not cap.isOpened(): print("ERROR: Cannot open webcam") exit() frame_w = int(cap.get(cv2.CAP_PROP_FRAME_WIDTH)) frame_h = int(cap.get(cv2.CAP_PROP_FRAME_HEIGHT)) # ── State variables — MUST be outside the loop ─────────────── # If declared inside the loop, they reset every frame. # closed_frames would never reach 20 — no alert would ever fire. closed_frames = 0 # counts consecutive frames below EAR threshold alert_count = 0 # total number of drowsy alerts in this session frame_count = 0 # total frames processed (for FPS) start_time = time.time() print(f"System ready. Frame: {frame_w}x{frame_h} ¦ Threshold: EAR<{EAR_THRESHOLD} for {DROWSY_FRAMES} frames") print("Starting detection loop... (press q to quit)")

### Part D — Main processing loop

# ── Main loop ──────────────────────────────────────────────── while True: ret, frame = cap.read() if not ret: break frame_count += 1 frame = cv2.flip(frame, 1) # Mirror mode # ── MediaPipe processing ───────────────────────────────── frame_rgb = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB) frame_rgb.flags.writeable = False results = face_mesh.process(frame_rgb) frame_rgb.flags.writeable = True # ── FPS calculation ─────────────────────────────────────── fps = frame_count / max(time.time() - start_time, 0.001) # ── Process if face detected ────────────────────────────── if results.multi_face_landmarks: face_lm = results.multi_face_landmarks[0] # Extract pixel coordinates for both eyes left_coords = get_eye_coords(face_lm, LEFT_EYE, frame_w, frame_h) right_coords = get_eye_coords(face_lm, RIGHT_EYE, frame_w, frame_h) # Calculate EAR for each eye left_ear = calculate_EAR(left_coords) right_ear = calculate_EAR(right_coords) # Average both eyes — robust to facial asymmetry avg_ear = (left_ear + right_ear) / 2.0 # ── Threshold logic — the core of drowsiness detection ── if avg_ear < EAR_THRESHOLD: # EAR below threshold — eye is closing or closed closed_frames += 1 # increment the danger counter else: # EAR above threshold — eye is open closed_frames = 0 # RESET counter on every open frame # Why reset to 0? A blink is normal. # Only SUSTAINED closure (20+ frames) means drowsiness. # ── Determine alert state ──────────────────────────── is_drowsy = closed_frames >= DROWSY_FRAMES # RED is_warning = 0 < closed_frames < DROWSY_FRAMES # YELLOW is_safe = closed_frames == 0 # GREEN # ── Choose eye contour colour by state ─────────────── eye_colour = RED if (is_drowsy or is_warning) else GREEN # ── Draw eye contours ───────────────────────────────── draw_eye_contour(frame, left_coords, eye_colour) draw_eye_contour(frame, right_coords, eye_colour) # ── Draw coloured frame border by state ─────────────── if is_drowsy: border_colour = RED border_thick = 8 elif is_warning: border_colour = YELLOW border_thick = 4 else: border_colour = GREEN border_thick = 2 cv2.rectangle(frame, (0, 0), (frame_w - 1, frame_h - 1), border_colour, border_thick) # ── DROWSY ALERT — red banner across top of frame ──── if is_drowsy: # Count unique alert events (each transition from warning to drowsy) if closed_frames == DROWSY_FRAMES: # exactly at threshold — new event alert_count += 1 print(f"*** DROWSY ALERT #{alert_count} at frame {frame_count} ***") # Draw solid red banner cv2.rectangle(frame, (0, 0), (frame_w, 60), RED, -1) # -1 = filled cv2.putText(frame, "DROWSY DETECTED — WAKE UP!", (10, 42), cv2.FONT_HERSHEY_SIMPLEX, 1.0, WHITE, 2) # ── HUD: EAR value, counter, state ─────────────────── # Positioned below any alert banner hud_y = 90 if is_drowsy else 30 cv2.putText(frame, f"EAR: {avg_ear:.3f} (L:{left_ear:.2f} R:{right_ear:.2f})", (10, hud_y), cv2.FONT_HERSHEY_SIMPLEX, 0.6, WHITE, 2) cv2.putText(frame, f"Closed frames: {closed_frames}/{DROWSY_FRAMES}", (10, hud_y + 28), cv2.FONT_HERSHEY_SIMPLEX, 0.6, (YELLOW if is_warning else WHITE), 2) state_text = "DROWSY" if is_drowsy else ("CAUTION" if is_warning else "SAFE") state_colour = RED if is_drowsy else (YELLOW if is_warning else GREEN) cv2.putText(frame, f"State: {state_text}", (10, hud_y + 56), cv2.FONT_HERSHEY_SIMPLEX, 0.6, state_colour, 2) else: # ── No face detected ───────────────────────────────── # IMPORTANT: Reset closed_frames when face disappears. # Without this: if counter was at 18 and face briefly # leaves frame, it holds at 18. When face returns, just # 2 more frames below threshold triggers a false alarm. closed_frames = 0 cv2.putText(frame, "No face detected", (10, 30), cv2.FONT_HERSHEY_SIMPLEX, 0.7, (0, 100, 255), 2) # ── FPS display ────────────────────────────────────────── cv2.putText(frame, f"FPS: {fps:.0f} ¦ Alerts: {alert_count}", (frame_w - 200, 30), cv2.FONT_HERSHEY_SIMPLEX, 0.55, (200, 200, 200), 1) # ── Show frame ─────────────────────────────────────────── cv2.imshow("Drowsiness Detector — Day 3", frame) # ── Print EAR to terminal every 30 frames ──────────────── if results.multi_face_landmarks and frame_count % 30 == 0: face_lm = results.multi_face_landmarks[0] l_coords = get_eye_coords(face_lm, LEFT_EYE, frame_w, frame_h) r_coords = get_eye_coords(face_lm, RIGHT_EYE, frame_w, frame_h) l_ear = calculate_EAR(l_coords) r_ear = calculate_EAR(r_coords) print(f"Frame {frame_count:5d} ¦ FPS: {fps:4.1f} ¦ ", f"L_EAR: {l_ear:.3f} ¦ R_EAR: {r_ear:.3f} ¦ ", f"AVG: {((l_ear+r_ear)/2):.3f} ¦ ", f"Closed: {closed_frames}") if cv2.waitKey(1) & 0xFF == ord('q'): break # ── Clean up ───────────────────────────────────────────────── face_mesh.close() cap.release() cv2.destroyAllWindows() print(f"Session complete. Frames: {frame_count} ¦ Total alerts: {alert_count}")

# 5. The Three Bugs — Read Before Coding

These are the three bugs that almost every student hits on Day 3. Read this section before you start coding. Recognising the symptom immediately saves hours of debugging.

## Bug 1 — closed_frames declared inside the loop

| ⚠ Bug | Symptom: EAR drops below threshold, counter increments to 1, then resets to 0 on the very next frame. Alert never fires no matter how long you close your eyes. |
| --- | --- |

# WRONG — resets to 0 every single frame: while True: closed_frames = 0 # ← THIS LINE IS THE BUG ret, frame = cap.read() ... if avg_ear < EAR_THRESHOLD: closed_frames += 1 # increments to 1, then resets to 0 next frame # CORRECT — declared once before the loop: closed_frames = 0 # ← declare HERE, outside the loop while True: ret, frame = cap.read() ... if avg_ear < EAR_THRESHOLD: closed_frames += 1 # now persists across frames: 1, 2, 3, ... 20

## Bug 2 — Normalised coordinates passed to EAR formula

| ⚠ Bug | Symptom: EAR always prints 0.01–0.04 even with eyes fully open. No alert ever fires because EAR is always below 0.25. |
| --- | --- |

# WRONG — landmark.x is 0.0-1.0, not pixel position: def get_eye_coords_WRONG(face_landmarks, eye_indices): coords = [] for idx in eye_indices: lm = face_landmarks.landmark[idx] coords.append((lm.x, lm.y)) # ← values like (0.512, 0.341) return coords # Result: euclidean_distance((0.512, 0.341), (0.621, 0.350)) # = sqrt(0.012 + 0.000) ≈ 0.11 — far too small # EAR = 0.11 / (2 * 0.15) = 0.03 — always below threshold # CORRECT — multiply by frame dimensions first: def get_eye_coords(face_landmarks, eye_indices, frame_w, frame_h): coords = [] for idx in eye_indices: lm = face_landmarks.landmark[idx] x_px = int(lm.x * frame_w) # ← 0.512 * 640 = 327 y_px = int(lm.y * frame_h) # ← 0.341 * 480 = 163 coords.append((x_px, y_px)) return coords

## Bug 3 — Counter not reset when face leaves frame

| ⚠ Bug | Symptom: Friend moves slightly out of frame during the demo, then back in — DROWSY alert fires immediately even though eyes were open. Embarrassing false alarm during presentation. |
| --- | --- |

# WRONG — missing the else reset: if results.multi_face_landmarks: # ... EAR calculation ... if avg_ear < EAR_THRESHOLD: closed_frames += 1 else: closed_frames = 0 # No else branch for when face is not detected! # closed_frames holds its last value — 18 — waiting to fire # CORRECT — always reset when face is lost: if results.multi_face_landmarks: # ... EAR calculation ... if avg_ear < EAR_THRESHOLD: closed_frames += 1 else: closed_frames = 0 else: closed_frames = 0 # ← THIS LINE prevents the false alarm cv2.putText(frame, "No face detected", (10,30), ...)

# 6. Testing Checklist — Before Day 4

|  | Test | Expected result |
| --- | --- | --- |
| 1 | Run ear_detector.py — unit tests first | Prints 'PASS' for euclidean_distance(), EAR test value ~0.28–0.32 |
| 2 | Look at webcam with eyes open | EAR displays ~0.28–0.35, state = SAFE, green border |
| 3 | Blink normally | Counter increments then resets to 0 — no alert |
| 4 | Close eyes and hold for 2 seconds | Counter climbs 1...2...3...20, red banner fires, alert prints to terminal |
| 5 | Open eyes after alert | Counter resets to 0, green border returns, state = SAFE |
| 6 | Move out of frame while counter is at 15 | Counter resets to 0 when face disappears |
| 7 | Move back into frame | Detection resumes immediately, no false alarm |
| 8 | Check terminal output | EAR values printing every 30 frames — verify left/right/avg values |
| 9 | Test with DroidCam (friend's face) | All above tests pass with remote face — same EAR thresholds |
| 10 | Screen record: full drowsy sequence | 30-sec clip: open → counter climbs → alert fires → open → resets |

# 7. Day 3 Journal — Write Tonight

Write in your own words. Do not copy from this document. These are real defense questions.

| Question | Your answer (write tonight) |
| --- | --- |
| Write the EAR formula from memory without looking |  |
| Why do you measure vertical distance at TWO points (P2-P6 and P3-P5) instead of one? |  |
| What does np.linalg.norm() compute internally? |  |
| Why is 20 frames chosen rather than 10 or 50? |  |
| Why do you average left and right EAR instead of using just one? |  |
| What EAR value did YOUR eyes produce when fully open? (write the actual number from terminal) |  |
| What happens to closed_frames if you declare it inside the while loop? |  |
| Why must closed_frames reset to 0 when the face leaves the frame? |  |
| Who introduced the EAR formula and in what year? |  |
| What is the key difference between your implementation and the original paper? |  |

# 8. Day 4 Preview — MAR (Yawn Detection)

Day 4 adds yawn detection. The Mouth Aspect Ratio (MAR) is the same concept as EAR but for the mouth — it detects when the mouth opens wide (yawning). The mouth landmark indices are already defined in today's code as MOUTH_IDX.

- Learn: Why yawning is a stronger early-drowsiness signal than eye closure

- Learn: MAR formula — same concept as EAR, different threshold (~0.65 for yawn)

- Code: calculate_MAR() function — identical structure to calculate_EAR()

- Code: Add yawn counter — separate from closed_frames counter

- Code: Combined drowsiness score — weighted average of EAR signal and MAR signal

- Code: Yawn count tracker — how many yawns in the last 60 seconds

- Journal: What is a physiologically appropriate MAR threshold for yawn detection?

| End of Day 4 goal | You will yawn in front of the webcam and see 'YAWN DETECTED' on screen. Your system now monitors two physiological drowsiness signals simultaneously — eye closure and yawning. That is a genuinely more robust detector than EAR alone. |
| --- | --- |

Day 3 Documentation — Drowsiness Detector — M.Sc. Software Engineering